# Joint-zone offset study: bullet notes

Side study to the portalframe partitioning comparison. Repo: railgnam/offset_parameter_study

## Objective

B1 vs B2: do the 300 mm timber stubs belong to the joint or to the main substructure?

Two points give no trend: build a curve

7 rungs D0..D6, offset t = 0, 30, 90, 150, 210, 270, 300 mm

Same mesh, wind-only load, coupling type and reference on every rung

Question: how far must the boundary sit from the joint?

## Method

Decks generated from B1/B2 by text transform (no CAE)

Stub = 10 layers of 30 mm, cuts split existing layers: identical mesh

Reference C25: full model, dowel-embedding radius 25 mm

Same result as old C to 1e-6 MPa

Checks

Reaction 60 kN on every rung

D6 reproduces B1 in all 102 region rows

Wall/slab load points move between substructures, none lost or doubled

## Results (deviation from C25)

Joint 6: error falls with offset, flat from about 210 mm

Timber S11 peak -19 % (t=0) -> -7 % (150) -> -4.5 % (300)

p95 -13 % -> -4.7 %

Joint 3 (load-carrying)

Peaks shrink: dowel Mises +31 % -> +12 %, timber S11 -9 % -> -0.5 %

Dowel mean drifts the other way: -6 % -> -12.5 %

Timber mean/p95 within +-3 % on every rung

Floor: -4.5 % (J6 timber), -12 % (J3 dowel mean) at t=300, independent of offset

Comes from substructuring itself, not the boundary position

Rule of thumb: 150-210 mm (5-7 layers) gets most of the gain, nothing beyond 210 mm

## Kinematic coupling probe (D3K, t=150)

Rigid interfaces at all cuts: solves, same reaction

Best J6 timber of all rungs (p95 -1.3 % vs -4.7 % at D6)

Mixed elsewhere: J3 dowel mean -13.9 % (worse), dowel peak +10.5 % (better)

One point only, not a series

## Published models

Mid joint coupling: distributing in B1, kinematic in B2; top/base kinematic in both

Published B2 also carries user and snow loads; B1 and C wind-only

B1 vs B2 mixes zone size, coupling type and load: this study isolates zone size

D0 much worse than published B2 at J6 (-19 % vs -0.6 % timber peak): load and coupling not yet separated

## If asked

J3 dowel mean worse with bigger zone?

Small zones add boundary error that partly cancels the floor; cause of the floor not established

Is J6 meaningful?

Lightly loaded, percentages amplify; use for trend shape, quote absolute values if asked

Inter-module plates?

About +82 % on every rung, same as published B1: edge effect from the main study

Cost?

Same mesh and DOF count on every rung; not a runtime study

## Open

Separate B2 load vs coupling effect (D0 with kinematic mid joint)

Back-calculation of the main substructures (deferred)

Full kinematic ladder if D3K holds up