# Joint-zone offset study: conclusions for the presentation

Side study to the portalframe partitioning comparison. Repo: railgnam/offset_parameter_study.

Numbers: compare/exports/2026-10-07_zone_offset_c25/tables/T10_zone_offset_trend.csv, figures F9.

## Objectives

B1 and B2 differ in one thing: whether the 300 mm timber stubs next to each joint belong to the joint substructure (B1) or the main one (B2). Two points cannot show a trend.

Question: how far from the joint must the substructure boundary sit before it stops disturbing the joint stresses?

Answer it as a curve: 7 rungs D0..D6 at offset t = 0, 30, 90, 150, 210, 270, 300 mm, moved together at the mid, top and base joints.

Keep everything else fixed: same mesh, same wind-only load, same coupling type, same reference.

## Method in one slide

Decks generated from the B1/B2 decks by text transform, no CAE. Each stub is 10 layers of 30 mm, so cuts split existing element layers: the mesh is identical on every rung.

Reference C25 = full model with dowel-embedding radius 25 mm (as in the substructures). It gives the same results as the old C to 1e-6 MPa.

Checks: total reaction 60 kN on every rung; D6 reproduces B1 in all 102 region rows; wall/slab load points (150 mm from the joint) move between substructures without being lost or doubled.

## Results (deviation from C25)

Joint 6: error shrinks steadily with offset and levels off by about 210 mm. Timber S11 peak −19 % (t=0) → −7 % (150) → −4.5 % (300); p95 −13 % → −4.7 %.

Joint 3 (load-carrying): peaks shrink with offset (dowel Mises peak +31 % → +12 %, timber S11 peak −9 % → −0.5 %), but the dowel mean drifts the other way (−6 % → −12.5 %). Timber mean/p95 stays within ±3 % on every rung.

Floor: about −4.5 % at J6 timber and −12 % at J3 dowel mean remain at t=300 and do not depend on the offset. That part comes from substructuring itself, not from the boundary position.

Practical rule: about 150–210 mm (5–7 element layers of 30 mm) captures most of the gain; beyond 210 mm nothing changes.

## Kinematic coupling probe (D3K, t=150)

Rigid interfaces at all cuts: solves cleanly, same reaction.

Best of all rungs at J6 timber (p95 −1.3 % vs −4.7 % at D6). Mixed elsewhere: J3 dowel mean −13.9 % (worse), J3 dowel peak +10.5 % (better).

Promising, one point only. Not a series yet.

## Finding about the published models

Interface coupling types differ: mid joint is distributing in B1 but kinematic in B2; top and base joints kinematic in both.

Published B2 also has user and snow loads; B1 and C are wind-only.

So published B1 vs B2 mixes three effects: zone size, coupling type, load. This study isolates zone size. D0 (t=0, distributing) is much worse than published B2 at J6 (−19 % vs −0.6 % timber peak); load and coupling type are not separated yet.

## Likely questions

Why does J3 dowel mean get worse with a bigger zone? Small zones add a boundary error that partly cancels the offset-independent error. Larger zones remove the cancellation. Cause of the floor not established.

Is J6 meaningful? It is lightly loaded, so percentages amplify small stresses. Use it for the trend shape, quote absolute values if asked.

Inter-module plates? About +82 % on every rung, same as published B1, independent of offset: the mesh-related edge effect from the main study.

Cost? Same mesh and same global DOF count on every rung; not a runtime study.

## Open

Separate B2's load vs coupling-type effect (D0 with kinematic mid joint).

Back-calculation of the main substructures (deferred).

Full kinematic ladder if D3K holds up.